# SYDNEY WATER

BUILDING PLAN APPROVED -

SUBJECT TO REQUIREMENTS

EITHER: Dolfin No:  ………………………………………	Quick Check Ref No:  …………………………

OR: Dolfin No: NOT REQUIRED IF …….…… Sydney Water Tap inTM Ref No: ……………..…………...

e-Developer Case No:  ……………………….

## Property Location

Street No:  ……………………………	Lot No:  ………………..

Street Name:  ………………………………………………………………………………………….

Suburb:  …………………………………………………………………………………………………

Building/Structure Description:  ………………………………………………………………………

Building Plan No:  ……………………………….	Engineers Plan No:  …………………………

Proposed building/structure is APPROVED to construct OVER/ADJACENT TO a Sydney Water asset, subject to the following requirements:

(NB. Delete non applicable requirements)

- The foundations/piers are to be founded below the zone of influence (OR the following to be used for lightweight structures) The foundations/footings are to be no less than a minimum of 600mm horizontal distance and 750mm vertical distance from the outside edge of the asset to the nearest face of the foundations/footings

- No part of the building/structure or its foundations to be less than a minimum ………… metre, horizontal distance from the outside edge of the asset.

- No part of the swimming pool or its foundations to be less than a minimum ………… metre, horizontal distance from the outside edge of the asset to the outer edge of the pool.

- No part of the above-ground building/structure to be less than 1m (for MH on mains up to DN300)/600mm(for MS, Lampholes, RP on mains up to DN300) horizontal distance from outside edge of any Sydney Water structure.

- No part of the below ground building/ structure/foundations to be less than 600/900mm horizontal distance from outside edge of any Sydney Water Structure. (900mm required for piering solutions)

- No part of any swimming pool (excluding coping) is to be less than 2m horizontal distance from outside edge of any Sydney Water structure.

- Foundations/piers are constructed in accordance with Engineers detail plans (stated above) as submitted to Sydney Water.

- All foundations/piers are to be founded to below the zone of influence or to solid rock.

- Indemnity letter to be signed by owner/s of property and returned to Water Servicing Coordinator prior to issue of building plan approval.

- Concrete encase approximately …………… metres of asset.  Concrete encasement to be carried out by an Accredited Constructor of Minor Works (Sewer) / Constructor and a Minor Works Agreement signed prior to commencement of works.

- Concrete encasement must extend a minimum of 1m past the external walls of the building/structure.

- Minimum of …………… mm vertical clearance between top of concrete encasement to underside of concrete slab.

- Property connection point (junction) to be inserted under Minor Works Agreement.

- All works are to be completed in accordance with Case No.  ……………

# SPECIAL REQUIREMENTS

(a)

(b)

(c)

NOTE:

Above requirements must be inspected/supervised by a WSC to enable the issue of a satisfactory compliance letter.

### APPROVED BY

### WSC Company Name:  ………………………………………………………………………

Name of Key Personnel: ……………………………………………………………………..

Signature of Key Personnel: ……………………………………….		 Date: ……../……../……..